Hydraulic Erosion and Sidewall Failure during Overtopping of a Cohesive Embankment: A Large-Scale Experiment

Meiman Zhang¹, Xu Yang², Jianjian Cui³, Xuan Huang⁴, Jianjun Han⁵, Jiang Qian⁶, Han Chen⁷, Jierong Chen⁸, Jin Xu⁹, Hai Zhu¹⁰, and Lingling Wang¹¹

Contents of this file:

- Text S1. Study area

- Text S2. Derivation and processing of hydraulic and erosion variables

- Figure S1. Location and geographical setting of the Yaotan Polder breach

- Figure S2. Laboratory apparatus used for geotechnical characterization

- Figure S3. UAV platforms and assigned observation tasks

- Figure S4. Oblique UAV acquisition and SfM photogrammetric reconstruction workflow

## Text S1. Study area

In 2020, the upper reaches of the Huaihe River experienced consecutive heavy rainfall events, leading to critical flood conditions in the Xuyi section. At 11:50 AM on August 12, a breach occurred at the Yaotan polder embankment within the flood diversion area of the main Huaihe River channel. The geographic coordinates of the breach are 32°58′23.7″N, 118°20′03.1″E. The study area of the Yaotan Polder is shown in Figure S1.

Figure S1. Location and geographical setting of the Yaotan Polder breach. (a) Location of the study region in China. (b) Topography and river network around Xuyi. (c) Location of the Yaotan Polder. (d) Aerial view of the breach site and the adjacent embankment.

## Text S2. Derivation and processing of hydraulic and erosion variables

S2.1 Reservoir water balance and discharge processing

Breach outflow was estimated from changes in upstream reservoir storage during active flow. For the ith observation interval, the effective horizontal reservoir area Aᵣ was 500 m² and the nominal interval Δtᵢ was 10 s. The reservoir-level increment and corresponding storage change were

where hᵣ,ᵢ is reservoir water level and Qin = 3.36 m³/s. The subscript r distinguishes reservoir water level from local flow depth h in the total-head calculation below. Reservoir drawdown gives a negative storage change and an outflow estimate greater than Qin.

Successive water-level increments were smoothed using a five-point centered moving average before calculating the processed outflow series. The window used the available increments from i − 2 to i + 2 within the same active-flow segment; shortened windows were used at segment ends. At the nominal 10-s interval, the full window represented approximately 50 s. Negative processed outflow estimates were set to zero. The five active-flow segments were processed independently, without averaging across controlled interruptions.

The calculated series therefore represented moving-window water-balance estimates rather than directly measured instantaneous discharge. Connecting the maximum estimate from each active-flow segment formed the segment-peak envelope used for stage-level comparison. Gate closure, drainage, surveying, refilling, and restart acceleration were excluded from the active-flow record according to the effective-time definition in the main Methods.

S2.2 DEM sampling and apparent erosion rate

Successive georeferenced DEMs were sampled along the breach channel at Δx = 0.5 m, following the workflow in main-text Fig. 3(a). At section i, positive bed-elevation loss Δzᵢ between successive surveys was divided by the intervening effective-flow duration:

The denominator followed the cumulative effective-flow time basis defined in the main Methods and excluded gate closure, drainage, UAV surveying, reservoir refilling, and the approximately 5–10-s acceleration period after each restart. Eᵢ was an interval-averaged apparent erosion rate. DEM differencing could include both gradual surface lowering and discrete elevation losses occurring between surveys; it did not separate continuous erosion from event-related material loss.

S2.3 Total head, energy slope, and estimated hydraulic stress

The hydraulic calculation used adjacent sections separated by Δx, as illustrated in main-text Fig. 3(b). Total head and energy slope were expressed as

where zᵢ is bed elevation, hᵢ is local flow depth, uᵢ is PIV-derived surface velocity, Rᵢ is hydraulic radius, ρ is water density, and g is gravitational acceleration. The surface velocity was used in the velocity-head term without a measured conversion to depth-averaged velocity. Because the breach flow was rapidly varied, τᵢ was treated as an estimated hydraulic index rather than a direct measurement of boundary shear stress.

S2.4 Normalization and fitting range

Only Stages 1–2 observations were used to fit the excess-shear relation. Writing the fitted slope as a gives the equivalent expressions

Here a denotes the Stages 1–2 fitted slope and is used only to make the normalization explicit; no additional numerical slope value is introduced. The normalization used the condition-specific fitted value τc = 27.56 Pa. Stage 3 observations were expressed in the same normalized space but were excluded from fitting. No independent erodibility measurement was used to obtain τc, and agreement of the fitting observations with the baseline was calibration rather than independent validation.

S2.5 Uncertainty sources and interpretation

Reservoir-level readings depended on staff-gauge graduation and image visibility. Differencing those readings and applying a moving window affected the temporal resolution of the discharge estimates. DEM registration and differencing, the discretely surveyed geometry, and the effective-flow duration affected E. Hydraulic estimates also depended on the reconstructed section geometry, water depth, total-head differences, and substitution of PIV surface velocity for depth-averaged velocity.

Uncertainty from DEM differencing, effective time, water-level reading, and hydraulic estimation was not jointly propagated to E* and τ*. Stage comparisons therefore describe directional departures without assigning statistical significance or quantifying the separate contributions of continuous erosion and discrete material loss. The single experiment, controlled interruptions, and discrete DEM/PIV sampling further bounded mechanism interpretation, as discussed in the main Limitations section.

## Figure S2. Laboratory apparatus used for geotechnical characterization

Figure S2. Laboratory apparatus used for geotechnical characterization of the field-sourced fill. (a) Fall-cone apparatus used to determine the Atterberg limits in accordance with GB/T 50123-2019. (b) Pycnometer and balance used to determine the specific gravity of soil solids.

## Figure S3. UAV platforms and assigned observation tasks

Figure S3. Unmanned aerial vehicle (UAV) platforms and assigned observation tasks. (a) DJI Mavic Air, used by UAV Groups 1 and 3 to record the fixed staff gauges and continuous oblique 4K video of headcut development, cracking, and block loss, respectively. (b) HKUAV-VFV-X4, used by UAV Group 2 to image surface tracers above the breach for particle image velocimetry. (c) DJI Phantom 4, used by UAV Group 4 to acquire multi-angle, high-overlap oblique photographs of the drained breach surface for topographic reconstruction.

## Figure S4. Oblique UAV acquisition and SfM photogrammetric reconstruction workflow

Figure S4. Oblique UAV acquisition and structure-from-motion (SfM) photogrammetric reconstruction workflow. (a) UAV Group 4 acquiring multi-angle, high-overlap oblique photographs of the drained breach surface along a preplanned flight path. (b) DJI Terra workspace used for image alignment, dense point-cloud generation, georeferencing with ground control points and surveyed reference elevations, and surface reconstruction. The resulting georeferenced point clouds and digital elevation models were used for stage-specific topographic differencing and profile extraction.